#### ANSWERS

#### Laboratory Exercise Manual

#### for

#### Designing with Quartus

Exercise 2 Part 2

| Device Name | EP20K100FC324-1 |
| --- | --- |
| Total Logic Elements | 116/4160 (2%) |
| Worst Fmax | 57.87 MHz (period = 17.281 ns) |
| Worst tsu and Pin Name | 20.166 ns dataa[1] |
| Worst tco and Pin Name | 13.003 ns sega |

- State Machine with Default Encoding

| - State Name | - State Encoding ¦code~7 ¦code~6 ¦code~5 ¦code~4 ¦code~3 |
| --- | --- |
| - Idle | - 00000 |
| - Lsb | - 00011 |
| - Mid | - 00101 |
| - Msb | - 01001 |
| - Err | - 10001 |

- Looking at the state machine encoding listed above, what type of state machine encoding does Quartus perform on this state machine?

- Answer: Pick one:	One-hot 	or 	Binary

- State Machine with Minimum Bits Assignment

| - State Name | - State Encoding ¦code~5 ¦code~4 ¦code~3 |
| --- | --- |
| - Idle | - 000 |
| - Lsb | - 001 |
| - Mid | - 011 |
| - Msb | - 100 |
| - Err | - 010 |

## Exercise 3 Fmax

| Worst Fmax | 121.71 MHz |
| --- | --- |
| Worst Fmax’s Clock Period | 8.216 ns |
| Register to Register Delay | 7.436 ns |
| Smallest Clock Skew | .000 ns |
| Micro Clock to Output | .330 ns |
| Micro Setup Delay | .450 ns |
| Solve the equation below with the above information: Clock Period (Worst Fmax) = Register to Register Delay – Smallest Clock Skew +Micro Clock to Output + Micro Setup Delay =(7.436 – 0.000 + 0.450 +_ .330) ns = 8.216 ns |  |

## Tsu

| Worst tsu | 11.332 ns |
| --- | --- |
| Longest pin to register delay | 13.184 ns |
| Micro setup delay | 0.450 ns |
| Shortest Clock Path to Destination Register | 2.302 ns |
| Solve the equation below with the above information: Setup Time (Worst tsu) = Longest pin to register delay – Shortest Clock Path to Destination Register + Micro Setup Delay = (13.184 – 2.302 + .450) ns = 11.332 ns |  |

## Tco

| Worst tco | 9.008 ns |
| --- | --- |

Exercise 4

Please summarize your results in this table as you complete the exercise:

| Source Name and Destination Name | ### Destination ### Clock | Required Longest ### P2P Time (ns) | Actual Longest P2P Time (ns) | Slack (ns) |
| --- | --- | --- | --- | --- |
| Datab_reg[2] to 12[6] | Clk1 | 9.220 | 7.255 | 1.965 |
| Dataa_reg[0] to 12[6] | Clk1 | 9.220 | 7.191 | 2.029 |
| Dataa_reg[0] to 11[3] | Clk2 | 2.553 | 6.298 | -3.745 |
| Datab_reg[0] to 11[6] | Clk2 | 2.553 | 5.887 | -3.334 |

What is the relationship between Required Longest P2P Time, Actual Longest P2P Time, and Slack?

____________Slack = Required max P2P Time – Actual max P2P time___________

Why does this design have two Required Longest P2P Times?

| Setting | Destination Clock | Required Setup Relationship |
| --- | --- | --- |
| No Multi-cycle Assignment | Clk2 | 3.333 |
| Multi-cycle = 2 | Clk2 | 16.666 |

## Exercise 5

## Table 1: Performance results

|  | FMAX (Mhz) |
| --- | --- |
| Step 1: Initial compile | 114.6 |
| Step 4: pipe_mult assigned to LogicLock region sub_module | 131.5 |
| Step 6: control assigned to new LogicLock region top_level | 132.1 |

Exercise 6

Figure 1

Figure 2